What to report about Buoys, Beacons and Daymarks.

Shapes, Sizes, and Colors of the Daymarks. - Square, green-colored shape with odd green-colored numbers that mark the left side of channel returning from the sea. - Triangle, red-colored shape with even red-colored numbers that mark the right side of the channel returning from the sea. - Octagonal, red and white colored shape with red-colored letters that mark the outer limit of a channel. - Always verify that the aid conforms to the IALA-B System, the Light List, and the nautical chart. Number or Letter on Daymark. [Per Light List and latest Nautical Chart] - Each aid usually has a number, occasionally a letter. - The color of the retroreflective material used for numbers and letters on daymarks will reflect the use of the aid. Nuns will have red characters and cans will have green characters. - Always verify that the charted symbol is correct. Is Daymark awash at any time? - Check and report the depth of water on the channel side of the aid only. Light colors and characteristics of Daymarks. - Light characteristics are the same as for lighted buoys. Verify the light characteristic that you observe to the characteristics listed in the Light List and printed on the latest nautical chart. - The visibility of a beacon will usually be greater than a buoy due to its height above MHW. - The range of visibility is shown on your nautical chart and in the Light List. - A daymark is considered to be a minor or small light when lighted. - Even if an aid is lighted, always check for lantern damage and/or vandalism. - Check that the solar panel is correctly oriented toward the sun. Structural condition of Daymarks. - Report all fixed aids that are leaning more than 15 degrees. - Report all aids that are knocked over. - Check for supports that are rotting from the top or bottom.. This problem is more visible at lower tides. Use your binoculars. - Are any structural pieces missing or broken? - Are any daymark panels missing? Purpose of the Daymark. - If a daymark is used as part of a range, are the panels obstructed in any way? Do the panels mark the center of the channel? - Are the panel colors correct as advertised in the Light List and on your nautical chart? - Is the color of the daymark faded? Growth of brush or structures near the Daymark. - Overgrowth of brush is usually corrected during scheduled annual Coast Guard maintenance. - Has the aid’s effectiveness been lessened by brush growth, tree growth, or new construction?