SCF - SMALL-CRAFT FACILITY Study Guide

TABLE OF CONTENTS

Introduction -2

SCF Study Guide Objectives – 2

Information on the SCF Program – 2

Visiting Small Craft Facilities – 3

Criteria for Visiting and Reporting a Small Craft Facility – 3

Small Craft Facility Listing for a Small Craft Chart – 4

Visits to a Small Craft Facility – 5

Guideline for a Small Craft Facility Updating visit - 5

USPS-NOS Cooperative Charting Program – Small Craft Facility Worksheet Instructions – 6

Heading Section – 6

Chart Info – 6

Location of the Facility – 6

Facility Availability – 6

Comments – 7

Facility Availability – 7

Attachments to your SCF Report – 8

Mailing Instructions – 9

Electronic Alternative Reporting – 9

NOAA Chart Order – 9

NOAA SCF Report Rejection - 9

INTRODUCTION:

An important element of the Auxiliary AN Program is SCF-Small-Craft Facility Updating. The U.S.C.G. Auxiliary members assist the National Oceanic and Atmospheric Administration (NOAA), National Ocean Service (NOS) in the accomplishment of this important mission. SCF activity involves the conducting of annual visits to marine facilities, located within the geographic area covered by a Small Craft Chart, for the purpose of updating small-craft facility services on SC Charts and other related publications. Small Craft (SC) charts and publications contain information that is important to small craft operators.

OBJECTIVES

1. 	To acquire a general knowledge of the responsibilities of the U.S.C.G. Auxiliary in the conduct of Small-Craft Facility Chart Updating patrols and mission activity.

2.	To become familiar with the procedure for conducting Small-Craft Facility Chart Updating patrols and mission activity.

3. 	To become familiar with the procedures for reporting SCF activity to AUXDATA.

INFORMATION ON THE SCF PROGRAM

The primary purpose for the Auxiliary’s participation in the Small-Craft Facility Updating program is specifically to assist the NOS-National Ocean Service with charted accuracy and with the updating of other NOAA publications.

NOS/NOAA charts are listed in the Nautical Chart Catalog which is available free from your local chart sales agent and also is available on-line from the National ATON Web Site. Among the charts listed in the National Chart catalogs are: Conventional Charts, Folio Small Craft Charts, Small-Craft (Pocket Fold) Charts, Other Small-Craft Charts and Marine Facility Charts. Small Craft-Charts are identified with (SC) after their chart number.

Small Craft Charts and Marine Facility Charts are specifically designed to provide detailed information for Small Craft operators. Additionally, some large-scale conventional charts list the locations of marine facilities along with the services these facilities provide for the small boat operator.  In addition, much of the data received on a Small Craft Facility Update report are incorporated into ENC—Electronic Nautical Charts.

The term SCF-Small Craft Facilities refers to marinas, boat yards and those facilities that provide services for small-craft and are open to the public.  These SCFs are normally found on NOS Small-Craft Charts for coastal waters / intracoastal waterways. However, other series of charts and documents show similar facilities including:

1) NOS Great Lake Charts (names only)

2) Coast Pilot

3) U.S. Army Corps of Engineers (USACE) river charts / appendixes (normally names only).

You should report small-craft facilities found on ALL NOS charts in addition to those specifically mentioned above.

Visiting Facilities

The easiest report to submit is the one for the marina at which you dock your boat. When you are cruising and stop at a marina, it is a fine opportunity to gather the information needed on that marina. Automobiles are frequently used to visit facilities because the cost of using a boat solely for this purpose is often prohibitive. Facility interviews by telephone is discouraged, since it does not permit inspection of the site and it makes it more difficult to get a business card or a letterhead.  Your visit to the SCF site adds credibility to your report and provides visual confirmation of the facility capabilities reported.

Criteria for Listing and Reporting a Small Craft Facility:

Not all marine facilities can become official Small Craft Facilities.  Basic criteria have been established, without which a marine facility is ruled out, or if currently listed, can be disqualified and removed from Small Craft Facility lists.  Note that data on marine facilities that do not meet the SCF criteria can still be reported as either a chart update or Coast Pilot update, or both.  Here are the basic qualifying criteria for a Small Craft Facility:

1. A small craft facility must be public (not private) and have, at a minimum, the following services:

Fuel sales to the public--Gasoline and / or Diesel.

A pump-out facility, or has local access to a municipality or community facility.

If there is a municipally managed pump out facility in the harbor, that this would fulfill this requirement.

Transient berths and/or moorings.  Transient means always available for a mariner who is traveling or making temporary stopovers.

2. The small craft facility must also have:

An approach depth.  This would be a navigable channel leading to the facility.

An alongside depth.  This would be sufficient water at the small craft facility’s dock or floats.

If the small craft facility fails to meet the above criteria, you may still report it:

- If it is currently listed on a small craft chart, and you are reporting it as a discontinuance, or

- You may submit the information as a Coast Pilot addition or revision.  Reference the section of Coast Pilot Reporting for the reporting procedure.

- You may survey the facility and report it as a Chart Update.

Small Craft Facility Listing from a Small Craft Chart

#### VISITS TO A SMALL CRAFT FACILITY

Small-Craft Facility activity can be conducted using an operational facility as part of an ATON patrol or on foot as an Operational Support Mission (Mission Code 41).  Generally, Auxiliary members prefer to conduct Small-Craft Facility activities as an ashore mission, using their privately owned vehicle or other conveyance.  Auxiliary members residing within close proximity of the marine facilities usually volunteer for these missions.  Also, there is a natural tie in to the Auxiliary PV Program.  Work with your FSO-PV or your SO-PV to become qualified.

PLEASE ONLY REPORT ON A SMALL CRAFT FACILITY ONCE A YEAR.

NOTE:  When not operating under Coast Guard orders, such as, a Safety patrol on an OPFAC, you must notify your FSO-AN, VFC or FC prior to starting your mission to insure that you have liability coverage.

GUIDELINE  FOR A SMALL CRAFT FACILITY UPDATING VISIT

Performing a SCF visit is considered a public appearance as a representative of the Coast Guard Auxiliary. Therefore, an Auxiliary member must be in proper uniform and must always act in a professional manner and be knowledgeable about the procedure for verifying a Small Craft Facility.

- Make the owner/manager of the marine facility the focus of your visit.

- Introduce yourself and provide a clear explanation of the purpose of the visit.

- Explain that you are participating in an Auxiliary program to update the Small Craft Marine Facility information in support of NOS/NOAA charts and other publications.

- Demonstrate to the owner/manager how the information about marine facilities is presented to the general boating public on NOS/NOAA charts, Coast Pilots, ENC and other publications.

- Show the Small Craft listings in the Small Craft Chart for the area.

- Show the references made in the Coast Pilot regarding small craft facilities.

- Indicate that other almanacs pick up information from government publications for their own publications.

- Review the criteria necessary to become and retain official Small Craft Facility status.

- Practice prudence in answering any technical questions regarding Aids to Navigation and Chart Updating subjects not covered within the report form.

- Auxiliarists are not allowed to perform measurements or other technical activity regarding the data that is reported.  Simply record the information that you receive as you receive it from the facility manager or POC—Person of Contact.

- If you cannot answer a technical or procedural question, discreetly refer the owner/operator to the nearest Coast Guard unit for an answer.

- If the question involves Small Craft Facility verifications, refer the question to your FSO-AN.

- Communicate back with the Facility manager or person of contact when you obtain an answer to their question(s).

- FSO-ANs should send copies of all questions to the DSO-AN who will prepare a “FAQ” publication that will become part of this procedure.

- Provide close out summary of the visit to the facility manager / POC.

- Review the information that you received and recorded on the SCF Form.

- Express appreciation for the level of cooperation that was provided.

USPS-NOS COOPERATIVE CHARTING  PROGRAM

SMALL CRAFT FACILITY WORKSHEET INSTRUCTIONS

# HEADING SECTION

# District - First Northern’s designation of “013” is pre-entered in this field. # Division - Enter your two-digit Division Number. (i.e., Division 3 = 03.) # Flotilla - Enter your two-digit Flotilla Number. # Report Number - Enter the year and the number for the report being submitted. Number your SCF reports so that NOAA can tell whether they missed a report. Your first report for 2007 would appear as “2007 001” # Member Emp No. - Enter your Auxiliary ID Number. # Last Name, First name and Initial - Enter your name as specified. # Street Address - Enter your full address as specified. # E-Mail Address - Enter your E-Mail address. # Telephone Number - Home number – enter the telephone number where you can be reached during normal business hours. Also, enter your business phone number. # Date of Investigation - Enter the date when the facility owner (POC) was interviewed. # Time of Investigation - Enter the time of day when the facility owner (POC) was interviewed. # Total Time Expended - Enter the total time expended, including travel time. Format using ‘h’ and ‘m’ as follows; 3 hours and 15 minutes is expressed as “3h15m.”

CHART INFO

# Chart  Number and Name - Enter the official number and name for the chart as referenced in the chart’s General Information Box.

Edition and Date - Enter the Date and Edition of the chart.  Normally listed at the bottom left-hand corner on the chart.

# LOCATION of the Facility

# Latitude - Enter the latitude taken at the facility in dd-mm-ss.ss N format.

# Longitude - Enter the longitude taken at the facility in ddd-mm-ss.ss W format. Plot the LAT/LON on the chartlet that you attach to your report.

# Locating Method Used - Indicate whether you used a GPS/WAAS or a DGPS to determine the FIX reported or it was provided by the owner / POC.

# EPE/HDOP - Indicate whether one of the following quality control readings are provided on your report: EPE or HDOP.

QC Reading - Indicate the actual EPE or HDOP reading provided by the GPS when the FIX was taken.

### FACILITY Availability ### Access to the Facility via land - Show any nearby major Federal or State Routes used to gain access to this facility by land vehicle. ### Facility Availability - Show a general expression as to the availability of the faqcility, such as, Open all year or Seasonal. ### Opens on: - If seasonal, show the month and date when the facility opens each year. ### Closes on: - If seasonal, show the month and date when the facility closes each year. ### Hours of Operation: - When open, indicate the hours when the facility is open for business, such as, 24 hrs, 0700 to 1100, etc. Use the 24 Hour Clock for time. # Days of Operation: - When open, indicate the days of the week when the facility is open for business, such as, 7 days, M thru F, etc.

### Comments

Enter any comments necessary to clarify your entries above. This is an optional field.  Many fields call for additional data. This is the location where that data should be reported. Always reference the field name for which the data is being provided.

### FACILITY Data

### Facility Number - This field is required on Facility Update and Discontinue a Facility reports.  Take the number from the SC Chart.

Facility Name - Required field on New Facility reports and optional on Facility Update reports.  Take the facility name from the SC Chart.  If a new report, use the corporate name for the facility.

Facility Manager - This field is required.  If there is no facility manager, enter the word “NONE.”  When ‘NONE” is entered, it is assumed that there will be a person of contact entry.

Person of Contact-POC - Optional field.  Use when the first contact at this facility is not the Facility Manager listed above or when there is no Facility Manager.

Facility Address - Required field. Enter the official mailing address for the facility.

City, State, and Zip Code. Enter the address data as specified.

Business Phone - Required field. Enter the phone number that is covered during normal business hours.

E-Mail Address - Enter the E-mail address for the POC.

FAX Number - Enter the FAX Number when available.

Web Site - Enter the web site address if available.

FACILITY Specifications

## Action - Check one box to define whether this report is a:

New Facility – Not currently listed on a Small Craft Chart.

Facility Update – Currently listed on a SC Chart.

Discontinue a SC Facility – Currently listed on a SC Chart but does not meet the criteria for a SCF.

SCF is a public facility. - Required/Criteria Field. YES / NO.  If NO is selected, a special message appears stating that this facility cannot become a SCF.  If it currently exists on a SC Chart, change the Action Code to discontinue.

Approach Depth at DATUM - Required/Criteria Field.  YES / NO.  If NO is selected, a special message  appears stating that this facility cannot become a SCF.  If it currently exists on a SC Chart, change the Action Code to Discontinue.  Depth should be obtained from the SCF POC.  Do not attempt to measure and report this depth on your own.  Depth should be corrected to local chart vertical Datum.

Alongside Depth at Datum - Required/Criteria Field.  YES / NO.  If NO is selected, a special message  appears stating that this facility cannot become a SCF.  If it currently exists on a SC Chart, change the Action Code to Discontinue.  Depth should be obtained from the SCF POC.  Do not attempt to measure and report this depth on your own.  Depth should be corrected to local chart vertical Datum.

Berths – transient - Required/Criteria Field.  YES / NO. If NO is selected, a special message  appears stating that this facility cannot become a SCF.  Transient berths are rented on a day to day basis.  List the number of transient berths available.  A NO response can be acceptable if there are transient moorings reported as available below.

Moorings – transient - Required/Criteria Field.  YES / NO. If NO is selected, a special message  appears stating that this facility cannot become a SCF.  Transient berths are rented on a day to day basis.  List the number of transient berths available.  A NO response can be acceptable if there are transient berths reported as available above.

Electricity - Required Field.  YES / NO.  If YES, indicate the amperage available in the comments section of this report.

Launching Ramp - Required Fields. YES / NO.  Show whether the ramp surface is natural or surfaced.

Repairs - Required Fields.  YES / NO.  Show the type of repairs performed at this facility—Hull, Motor, and/or Radio.

Lift Capacity - Required Field.  YES / NO.  Show the lift capacity in Tons.  Indicate the type of lift in the comments section of this report.

Boat Rental - Required Fields.  YES / NO.  Show the type of boat rental that is available—Canoe, Row, Motor, Charter, House, and/or Sail.

Food - Required Field.  YES / NO.  If the store is not on the premises, indicate the distance in the comments section of this report.

Lodging - Required Field.  YES / NO.  If the hotel or inn is not on the premises, indicate the distance in the comments section of this report.

Camping - Required Field. YES / NO. If the camp site is not on the premises, indicate the distance in the comments section of this report. Toilets - Required Field. YES / NO. Showers - Required Field. YES / NO. Laundry - Required Field. YES / NO. If the laundry is not on the premises, indicate the distance in the comments section of this report. Pump Out Station - Required/Criteria Field. YES / NO. If NO is selected, a special message appears stating that this facility cannot become a SCF. If it currently exists on a SC Chart, change the Action Code to discontinue. SCF must have a pump-out facility or have access to a municipality or community facility. Winter Boat Storage – Wet – Dry – Inside - Required Field. YES / NO. Wet storage is in the water. Dry storage is not in the water. Inside storage is in a building, out of the weather. Nautical Chart Sales - Required Field. YES / NO. Water - Required Field. YES / NO. Fresh water it available for transient slips. Showers - Required Field. YES / NO. Available for transient mariners. Ice - Required Field. YES / NO. Available for sale. Groceries - Required Field. YES / NO. Available for sale. If the groceries are not sold on the premises, indicate the distance in the comments section of this report. Hardware - Required Field. YES / NO. Available for sale. If the hardware is not sold on the premises, indicate the distance in the comments section of this report. Bait - Required Field. YES / NO. Available for sale. If the bait is not sold on the premises, indicate the distance in the comments section of this report. Tackle - Required Field. YES / NO. Available for sale. If the tackle is not sold on the premises, indicate the distance in the comments section of this report. Diesel Oil - Required/Criteria Field. YES / NO. If NO is selected, a special message appears stating that this facility cannot become a SCF. If it currently exists on a SC Chart, change the Action Code to discontinue. SCF must have a fuel capability of Diesel or Gasoline. Gasoline - Required/Criteria Field. YES / NO. If NO is selected, a special message appears stating that this facility cannot become a SCF. If it currently exists on a SC Chart, change the Action Code to discontinue. SCF must have a fuel capability of Diesel or Gasoline. VHF Channel monitored. - Required Field. YES / NO. If YES, indicate the specific channel that is monitored by this facility. Towing - Required Field. YES / NO. If YES, provide instructions in the Comments for getting in touch with the available towing. ATTACHMENTS to your SCF Report

#### Business Card - Attach a business card from the facility owner or POC. This adds credibility to your report. Indicate the quantity attached. #### Brochure for facility - If available, attach a brochure for the facility or for any of the special facilities at or near the facility. Indicate the quantity attached. #### Chartlet - Attach a chartlet showing the location of the facility. You can also submit a layout of the facility with LAT/LON as a Chart Update. #### Photographs - Indicate the number of photos attached to this report.

The ideal situation would be for the SCF reporter to be able to download this form to their own PC and perform the data input.  Set up a master with data in certain fields such as those on the “Submitted By” section. This will save you time when preparing your SCF reports.  Open and rename your SCF Worksheet each time you submit a report.

Mailing Instructions:

Combine all of the SCF Worksheet Report with any business cards, brochures, chartlet, and any photos in an envelope and mail to:

Frank Larkin

DSO-AN 013

107 Concord Road

Acton, MA 01720.

Always keep a copy for your own records.

Electronic Alternative Reporting:

Depending on your skill, you can avoid having to mail your completed report to the DSO-AN by using e-mail as follows:

Step 1 - Scan the SCF’s business card, brochures, and chartlet into your PC.

Step 2 - Address an e-mail to the DSO-AN at FrankJLarkin@verizon.net.

Step 3 – Attach a copy of the (EXCEL) SCF Workshop Report to the E-mail.  Attach copies of any scanned in business card, brochure, chartlet and any photos to the E-mail.

Step 4 – Send the E-mail to the DSO-AN.

NOAA Chart Order

A copy of the NOAA Chart may be ordered when the following conditions exist:

- A section of a nautical chart is attached to the SCF Report. - The SCF-Small Craft Facility meets all of the criteria for a legitimate Small Craft Facility. - The SCF report is professionally repaired, complete and acceptable to NOAA. NOAA SCF Report Rejection NOAA will return all the copies of rejected SCF Reports to the DSO-AN who will notify the Auxiliarist reporter of the problems noted by NOAA. This activity is not meant as punitive but more as a learning exercise so that the reporter will be able to submit correct reports in the future.