Non-Permitted PATON Worksheet Instructions

(7/4/07)

Date – Enter the date when the non-permitted aid was observed.

Area – Enter the name of the general area where the aid is located.

Prepared by:  Enter your name.

USCGAUX Affiliation:  Enter your District, Division, and Flotilla numbers.

Time – Enter the time when the aid was observed.

PATON Name – Enter the name of the type of aid that is observed.

Starboard Side Lateral Aid

Port Side Lateral Aid

Regulatory Aid

Special Purpose Aid

Light, Small

Light, Major, etc.

Latitude – Enter the latitude in DD-MM-SS.SS N format.

Longitude – Enter the longitude in DDD-MM-SS.SS W format.

Note: Follow the guidelines for taking fixes at aids to navigation. Show EPE or HDOP along with the Model No. and Manufacturer of the GPS used. Indicate whether WAAS or DGPS was enabled. Refer to the USCGAUX Federal Short Range Aid to Navigation Study Guide that is available at the AN Web Site at www.uscgaan.com.

USE – Show the purpose for the aid, such as, “Marks the channel,” “No Wake Regulations,” “Speed Regulations,” etc.

Type – Show the type of aid.  Typical abbreviations are:

ULB – Unlighted Buoy

LB – Lighted Buoy

LT – Light

DBN – Daybeacon, etc.

Depth at Datum – Enter the depth, corrected to DATUM.

Observed depth,

Plus	Correction for the location of Transducer,

Minus	Height of Tide,

Equals	Depth at Datum.

Note: Follow the guidelines for taking depths alongside aids to navigation. Show type of instrument used to take depth. Show the Height of Tide. (available from the Almanac screen on your GPS. Refer to the USCGAUX Federal Short Range Aid to Navigation Study Guide that is available at the AN Web Site at www.uscgaan.com.

Time Taken – Enter the time when the depth was taken.

Sound Signal – Answer yes or no.  If yes, show the type of sound signal.  If an electronic signal, indicated the blast characteristics.

Bell

Gong

Horn

Fixed / Floating – Indicate whether aid is floating, such as a buoy, or fixed, such as a structure.

Material – Show the material that the aid is made from, such as, foam, plastic, metal, wood, etc.

PATON matches the IALA-B System? –Answer yes or no.

PATON is currently deployed?  - Answer yes or no.

Physical PATON is watching properly?  Answer yes or no.

Does aid have a light? – Answer yes or no.

Light Characteristic – If lighted, enter the characteristic that list is showing.

Light Period – If lighted, enter the light period for the aid.

Light Color – If lighted, enter the color of the light.

Aid Duration – If aid is seasonal, enter the “from” and “to” dates for when the aid is deployed each year.

Local Authority – Enter the harbormaster or other local agency that manages the waterway in the area where the aid is located.

Owner – Enter the name of the owner of the aid.

Phone Number – Enter the phone number for the owner of the aid.

E-Mail Address – Enter the e-mail address of the owner.

POC – Enter the name of the Person of Contact.

Address – Enter the business address for the owner.

Discrepancies observed and equipment notes:

List any discrepancies observed on the aid. Show the GPS’s model number and manufacturer.  Show the EPE or HDOP recorded on scene when the fix was taken.Show the equipment used to take the sounding.  If an echo sounder is sued, show the model number and manufacturer.  Show the correction for the location of the transducer.  Show the Height of Tide for the time when the depth was taken.

Sample Worksheet

| DATE | 7/4/07 | PATON - NON-PERMITTED AID WORKSHEET |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AREA | WEYMOUTH BACK RIVER |  |  |  |  |  |  |
| PREPARED BY: |  | Frank Larkin |  | Affiliation: |  | 013-10-07 |  |
| Time |  | PATON Name |  |  |  |  |  |
| 1216 | REGULATORY BUOY |  |  |  | PATON |  | Physically |
| Latitude | Longitude | Use |  | Type | matches | PATON is | PATON is |
| 43-23-56.90 N | 070-23-45.69 W | NO WAKE REGULATIONS |  | LB | IALA-B | currently | watching |
| Depth at DATUM (ft) | Time Taken | Sound Signal? | Fixed / Floating | Material | System? | deployed? | properly? |
| 34 | 1220 | NO | FL | FOAM | YES | YES | YES |
| Does aid have a light? | Light Characteristic | Light Period | Light Color | Light Height (ft) |  | Aid Duration |  |
| YES | Fl | 4s | YELLOW | 5 | 15-Apr | to | 30-Nov |
| Local Authority | WEYMOUTH HARBORMASTER |  | Owner | JONES MARINA INC |  |  |  |
| Discrepancies observed and equipment notes: |  |  | Phone Number |  | E-Mail Address |  |  |
| GPS 76 by Garmin with WAAS enabled was used. |  |  | 978-0233-3345 | Jonesmarina@aol.com |  |  |  |
| EPE was 7.6 ft. A Hummingbird Wide 100 echo sounder |  |  | POC | JOHN JONES |  |  |  |
| was used to take depth, Height of Tide was 3.5 ft. |  |  | 24 Main Street |  |  |  |  |
| 39 + 0.8 - 3.5 = 34.3. Charted depth is 35 feet. |  |  | Weymouth MA 01239 |  |  |  |  |

Send a copy of this document as an attachment to an e-mail to:

ANT Chief.

ANT Auxiliary Representative.

ADSO-AN for your area.

Note: If you could get all of the information requested above, it would be great. Provide as much data as possible. Remember, the only person who can correct the non-permitted situation is the owner of the aid, and, the only way for the Coast Guard can get in touch with this owner is by the information that you provide. Due to time constraints, it is often difficult for the Coast Guard personnel to search the internet for information. Your role in getting the non-permitted aids in your area properly documented is to provide as much information as possible to the Coast Guard on this report.